Displaying The Windows of the 2001

Under normal usage of the 2001, a user would use the :SENSE:DATA? to return

a reading from the 2001. Using this command only returns the data in the

primary window. It does not return the units nor does it return any of the

information from the secondary display. If a programmer chooses to return

the data from the secondary window, he or she would have to program the 2001

to measure the very specific paramters. For example, if the 2001 is set for

DC volts and the next button was pushed, the display may look as follows:

+064.8283  mVDC

+000.405  mVAC           +060.15      Hz

In this case, only the value +64.8283E-03 would be returned to the computer.

To return the entire display, the command :DISPLAY:WINDOW1:DATA? is sent to

the 2001 to return the first window display and :DISPLAY:WINDOW2:DATA? is

sent to return the second window display. As shown in the example program

below, both windows can be returned:

If meter_enabled Then

cmd$=":DISPLAY:WINDOW1:DATA?"

Call Send(GPIBAddress,cmd$,status%)

If status% = 0 Then

Call Enter(disp1$, 21, l%, GPIBAddress, status%)

If status% = 0 Then

cmd$=":DISPLAY:WINDOW2:DATA?"

Call Send(GPIBAddress,cmd$,status%)

If status% = 0 Then

Call Enter(disp2$, 33, l%, GPIBAddress, status%)

If status% = 0 Then

Print disp1$

Print disp2$

End If

End If

End If

End If

End if

Please note in each Call Enter commands the values 21 and 33. The top

window of the 2001 can display 20 characters and the bottom window can

display 32 characters. When the window string is returned to the host

computer, all 20 and 32 characters are returned along with an additional

line feed (LF) character. Thus, to properly transfer the entire window

string, 21 and 33 characters must be returned, otherwise, an error condition

will occur. When both window strings have been transferred, the information

can be displayed on the CRT or an application program can separately parse

the information for the numerical data only for future use.

John Tucker   5/1/92